Scanner 2D Development Instruction

- Any applications need to receive data only without controlling the scanner, please do directly by the following steps:

  - Enable HWSoftScan, change the ‘Input Configuration’ to ‘Broadcast mode;

  - Register broadcast to receive the data in the customer’s app by referring to the HWSoftScan demo source codes.

For the details, please refer to HWSoftScan demo source codes.

2. Any applications need to control the scanner, please refer to the following methods or assist ScanUtil.java in HWScanTest to initial/call or control the other settings(please do not modify the parameter type and name)

Enable the scan service：

private final String ACTION_SCAN_INIT = "com.rfid.SCAN_INIT";

Intent intent = new Intent();

intent.setAction(ACTION_SCAN_INIT);

context.sendBroadcast(intent);

Set the data output method(broadcast mode as an example)

Intent intent = new Intent();

intent.setAction(ACTION_SET_SCAN_MODE);

// mode is an int value, 0 -- BroadcastReceiver mode; 1 -- Focus input mode (default)

intent.putExtra("mode", 0);

context.sendBroadcast(intent);

Set scan timeout value(5s by default)

Intent intent = new Intent();

intent.setAction(ACTION_SCAN_TIME);

// timeout is a String, Value:1000,2000,3000,4000,5000,6000,7000,8000,9000,10000

intent.putExtra("time", 3000);

context.sendBroadcast(intent);

Call Scan

private final String ACTION_SCAN_CMD = "com.rfid.SCAN_CMD";

Intent intent = new Intent();

intent.setAction(ACTION_SCAN_CMD);

context.sendBroadcast(intent);

Disable the scan service(scanner would not work unless enabling again)

public static final String ACTION_CLOSE_SCAN = "com.rfid.CLOSE_SCAN";

Intent toKillService = new Intent();

toKillService.setAction(ACTION_CLOSE_SCAN);

context.sendBroadcast(toKillService);

Data receiver method same as the above method, registering broadcast to receive it.