UHF API Instructions

- Copy .jar and .so to your project：

- Call UhfReader.class methods：

| 1.4 | UHF-ISO1800-6C | UhfReader |  |
| --- | --- | --- | --- |
|  | 1.4.1 | getInstance() |  |
|  | 1.4.2 | getFirmware() |  |
|  | 1.4.3 | setOutputPower() |  |
|  | 1.4.4 | inventoryRealTime() |  |
|  | 1.4.5 | selectEPC() |  |
|  | 1.4.6 | readFrom6C() |  |
|  | 1.4.7 | writeTo6C() |  |
|  | 1.4.8 | setWorkArea() |  |
|  | 1.4.9 | getFrequency() |  |
|  | 1.4.10 | setFrequency() |  |
|  | 1.4.11 | setAutoFre() |  |
|  | 1.4.12 | lock6C() |  |
|  | 1.4.13 | close() |  |

1.4  UHF-ISO1800-6C

1.4.1 getInstance()

| Function | UhfManager getInstance() |
| --- | --- |
| Description | Get Uhf manager instance and open device |
| a |  |
| Parameter | void |
| Return | Uhf manager instance |

1.4.2 getFirmware()

| Function | byte[] getFirmware() |
| --- | --- |
| Description | Get firmvare version |
| a |  |
| Parameter | void |
| Return | Version information |

1.4.3 setOutputPower()

| Function | boolean setOutputPower(int value) |
| --- | --- |
| Description | Set antenna power gain（The module will automatically set the power to the maximum each time it is powered on） |
| a |  |
| Parameter | Power value |
| Return | True: set success False: set failure |

Note: Scope of value, 16 - 26.

1.4.4 inventoryRealTime()

| Function | List<byte[]> inventoryRealTime() |
| --- | --- |
| Description | Real-time get uhf epc list |
| a |  |
| Parameter | void |
| Return | Epc list |

1.4.5 selectEPC()

| Function | void selectEPC(byte[] epc) |
| --- | --- |
| Description | Select epc(Before you can read or write a label, you need to call this method and select the label) |
| a |  |
| Parameter | epc |
| Return | void |

1.4.6 readFrom6C()

| Function | byte[] readFrom6C(int memBank, int startAddr, int length, byte[] accessPassword) |
| --- | --- |
| Description | Read tag data |
| a |  |
| Parameter | Membank, 0--reserve，1--epc，2--tid，3--user start address, unit: word read length, unit: word password, access password |
| Return | Tag data |

1.4.7 writeTo6C()

| Function | boolean writeTo6C(byte[] password, int memBank, int startAddr, int dataLen, byte[] data) |
| --- | --- |
| Description | Write data into iso-1800-6c tag |
| a |  |
| Parameter | Password, access password Membank, 0--reserve，1--epc，2--tid，3--user start address, unit: word data length, unit: word data |
| Return | True: write success False: write failure |

1.4.8 setWorkArea()

| Function | int setWorkArea(int area) |
| --- | --- |
| Description | Set device work area |
| a |  |
| Parameter | Work area 1->China2, 2->USA, 3->Europe, 4->China1, 6->Korea |
| Return | 0:set success Other: set failure |

Note: American standard by default. Generally do not need to be modified.

1.4.9 getFrequency()

| Function | int getFrequency() |
| --- | --- |
| Description | Get module work frequency |
| a |  |
| Parameter | void |
| Return | Frequency |

1.4.10 setFrequency()

| Function | int setFrequency(int startFrequency, int freqSpace, int freqQuality) |
| --- | --- |
| Description | Set frequency(Not recommended) Before calling this function, you need to call setWorkArea first, set the working area, and then set the frequency. |
| a |  |
| Parameter | Start Frequency, the starting frequency frequency Space, frequency interval frequency Quality, frequency points |
| Return | 0:set success Other: set failure |

Note: American standard by default. Generally do not need to be modified.

1.4.11 setAutoFre ()

| Function | boolean setAutoFre(boolean isOpen) |
| --- | --- |
| Description | Enable FHSS mode or not |
| a |  |
| Parameter | isOpen : true--open FHSS false--close FHSS |
| Return | true success false set failure |

1.4.12 lock6C ()

| 函数 | boolean lock6C(byte[] password, int memBank, int lockType) |
| --- | --- |
| 说明 | Lock the tag。 |
| a |  |
| 参数 | password, access password memBank, lock area 0-- kill password，1-- access password，2--epc，3--tid，4--user lockType, 0—open，1—open forever，2—lock，3—lock forever |
| 返回 | 0:set success Other: set fail |

Note：1. After permanent opening and permanent locking, the tag will not be able to change the lock type  2. To do the locking, you need to lock the kill password and access password areas first

1.4.13 close()

| Function | void close() |
| --- | --- |
| Description | Close device |
| a |  |
| Parameter | void |
| Return | void |

| Documentation time | Author | Version |
| --- | --- | --- |
| 2018-08-09 | HL | V1.2 |
| 2019-05-16 | HL | V1.3(Add ) |