VD-67

RFID desktop reader

Firmware upgrade instruction

Catalogue

1	Installation DfuSe_Demo_V3.0	4

2	fireware upgrade	5

# Installation DfuSe_Demo_V3.0

Step one：Double click to install DfuSe_Demo_V3.0_Setup.exe

。

Step two：select Modify，click next。

Select the defaults。

Step three：Finish installation。s

# Fireware upgrade

Step one：Let VD-67 desktop reader into DFU upgrade mode.

Connected the USB wire on computer.after that press keyboard emulation button then put the USB cable into Desktop reader.now is in DFU upgrade mode.

Step two ：Install the DFU drive。

- Click next ，To search the upgarde file.

- Click next ,According to instruction to find  STTub30.sys。

Select STTub30.sys file，Click ok

Finish installation。

Step three：Open DFU upgrade software（Search DfuSe->DfuSe Demonstration）。

Select the upgrade file。

Click【Upgrade】

Select 【是】。

Upgrade successful。